Stratum-1 Time Server

Web Dashboard

Requirements & Design Specification

v0.5 — FINAL

| Hardware | Raspberry Pi 5 + GNSS module (Waveshare Pico-GPS-L76X HAT / LC76G) |
| --- | --- |
| NTP daemon | chrony |
| GNSS interface | gpsd + NMEA sentences |
| PPS | GPIO PPS signal to chrony |
| Status | Pre-development — requirements phase |
| Notifications | v2 (out of scope for this document) |
| Auth / access control | None — trusted home network assumed |

# 1. Purpose and Scope

This document defines requirements, page structure, data sources, configurable parameters, and design assumptions for a self-hosted web dashboard reporting on a Raspberry Pi 5 Stratum-1 NTP time server. The dashboard is intended for home lab use, viewed on a mix of desktop and mobile browsers.

The system collects metrics from chrony (NTP discipline), gpsd / NMEA (GNSS receiver), and Pi system health. Data is polled on a configurable interval, stored in SQLite for historical trending, and served as JSON to static HTML pages rendered in the browser.

# 2. System Architecture

## 2.1 Components

| Component | Description |
| --- | --- |
| Backend poller | Python process running on the Pi. Polls chronyc, gpsd/NMEA, and system stats at a configurable interval. Writes raw samples to SQLite. |
| SQLite database | Single file on disk. Stores raw samples, pre-aggregated rolling averages, and configuration. Retention is configurable. |
| Web server | Lightweight HTTP server (e.g. Flask or nginx + uWSGI) running on the Pi. Serves static HTML/CSS/JS and JSON API endpoints. |
| Frontend pages | Six static HTML pages using Tabler (Bootstrap 5-based). Charts via ApexCharts. Pages poll JSON endpoints via fetch() on a configurable auto-refresh interval. |
| Config store | config.json on disk. Human-editable. Loaded at poller startup and by the config UI page. |

## 2.2 Data flow

- Poller reads: chronyc tracking, chronyc sourcestats, gpsd socket or NMEA serial, /proc and /sys for system stats

- Each poll cycle writes one raw sample row per source to SQLite

- SQLite views or a lightweight aggregation step maintain rolling averages (1 min, 5 min, 1 hr windows)

- API endpoints query SQLite and return JSON — instantaneous latest, rolling averages, and windowed history

- Browser pages call API endpoints every N seconds (configurable, default 10 s) and update the DOM

## 2.3 Technology choices

| Concern | Choice / Rationale |
| --- | --- |
| CSS framework | Tabler (built on Bootstrap 5). Chosen for its dashboard-first component set, built-in dark mode, responsive grid, and familiarity with Bootstrap conventions. |
| Charting | ApexCharts. Handles real-time updates, has polar/radial chart types (sky plot), good mobile rendering, MIT licence. |
| Backend language | Python 3. Already present on Pi OS, good chrony/gpsd library support. |
| Database | SQLite via Python sqlite3 stdlib. No server process, single file, easy backup. |
| Config format | JSON file (config.json). Separate from metrics DB; human-editable if UI is unavailable. |

# 3. Data Collection and Smoothing

## 3.1 Rationale

Raw PPS offset values from chrony can vary between approximately 2 ns and 200 ns on a well-disciplined server. Displaying a raw snapshot on a stat card gives a misleading picture of server quality. A rolling average provides a stable, honest headline figure, while retaining the raw trace on charts preserves diagnostic value.

## 3.2 Ring buffer and storage

- The poller maintains an in-memory ring buffer of the last 500 raw samples per metric for fast rolling calculations

- Every raw sample is also persisted to SQLite for long-term trending

- SQLite rows include: timestamp (Unix epoch), source (chrony / gnss / system), metric name, raw value

- A configurable retention policy deletes rows older than N days (default 90). Deletion runs on poller startup and nightly.

## 3.3 Aggregation windows

| Window | Use | Notes |
| --- | --- | --- |
| Instantaneous | Latest raw sample | Used for diagnostic displays and raw chart trace |
| 1-minute rolling mean | Stat card secondary value | Mean of last 6 samples at 10 s poll |
| 5-minute rolling mean | Stat card primary display value | Default headline figure on overview and detail pages |
| 10-minute rolling mean | Available on config | Optional, user can select preferred window |
| Hourly / daily aggregates | History page charts | Pre-computed min / max / mean per hour stored in SQLite |

## 3.4 Chart display

- Time-series charts show both the raw trace (thin, low opacity) and a smoothed overlay (bold line)

- The smoothing window used for the chart overlay is user-configurable (default 5 minutes)

- Configurable time windows: 1 hr / 6 hr / 24 hr / 7 d / 30 d

- Default chart window on first load is configurable (default 1 hr)

# 4. Page Specifications

## Page 1 — Overview

Purpose: answer 'is everything OK?' in under 3 seconds. Single-glance health summary with sparklines.

### 4.1.1 Stat cards (top row)

- System status — single colour badge (green / amber / red) derived from threshold evaluation across all key metrics

- PPS offset — 5-minute rolling mean, unit ns, with min/max range indicator for the same window

- Jitter — 5-minute rolling mean in ns

- GNSS fix — fix type label (No fix / 2D / 3D / 3D+PPS) + satellite count used/visible

- Stratum — current chrony stratum (should always be 1)

- Uptime — Pi system uptime

### 4.1.2 Sparklines

- PPS offset — last 60 minutes, raw trace only, small inline chart

- Jitter — last 60 minutes

- CPU temperature — last 60 minutes

### 4.1.3 Status detail panel

- Reference source name (friendly label from config, e.g. 'GPS PPS')

- Last updated timestamp (data age — goes amber if stale > 2x poll interval)

- Chrony reference ID and stratum

- Active threshold alerts listed (any amber/red conditions)

### 4.1.4 Behaviour

- Auto-refreshes on configurable interval (default 10 s)

- Last-updated indicator ages visibly if refresh fails

## Page 2 — NTP / Chrony Detail

Purpose: full chrony discipline picture. For diagnosing offset trends, frequency drift, and peer health.

### 4.2.1 Tracking panel

| Field | Source / notes |
| --- | --- |
| Reference ID | chronyc tracking — Reference ID field |
| Stratum | chronyc tracking — Stratum |
| Reference time | chronyc tracking — Ref time (UTC) |
| System time offset | chronyc tracking — System time |
| RMS offset | chronyc tracking — RMS offset |
| Frequency error | chronyc tracking — Frequency (ppm) |
| Residual freq | chronyc tracking — Residual freq |
| Skew | chronyc tracking — Skew (ppm) |
| Root delay | chronyc tracking — Root delay |
| Root dispersion | chronyc tracking — Root dispersion |
| Update interval | chronyc tracking — Update interval |
| Leap status | chronyc tracking — Leap status |

### 4.2.2 Sources table

Full output of chronyc sourcestats rendered as a sortable table with columns: Name, Mode (^/*/?), Poll, Reach, LastRx, Last sample offset, RMS offset, Std dev, Frequency, Skew. PPS source row highlighted. GPS source row includes tooltip explaining expected ~34ms NMEA processing offset. Peers with zero samples (e.g. unreachable sources such as fritz.box) must render gracefully — display dashes rather than zeros for unmeasured fields.

### 4.2.3 Charts

- System time offset over time — raw trace + smoothed overlay, configurable window

- Frequency correction (ppm) over time — reveals temperature-driven drift

- Offset histogram — distribution of offset values over selected window, shows disciplining quality

## Page 3 — GNSS Status

Purpose: receiver health and position/fix quality. Not the satellite sky view (that is Page 4).

### 4.3.1 Fix status panel

- Fix type indicator — No fix / 2D / 3D / 3D+DGPS / 3D+PPS (colour coded)

- HDOP, VDOP, PDOP values with threshold colouring

- Satellites used in fix / satellites in view

- PPS signal — present / locked / missing indicator

- GNSS time vs system clock delta

### 4.3.2 Position panel

Note: display of position is configurable (can be hidden — see Section 5 config). When shown:

- Latitude, longitude (decimal degrees, 6 d.p.)

- Altitude (metres)

- Position accuracy estimate if available from receiver

### 4.3.3 Constellation panel

- Breakdown of satellites by constellation: GPS (L1), GLONASS, Galileo, BeiDou — as reported by LC76G

- Count visible and count used per constellation

### 4.3.4 NMEA health panel

- Sentence rate (sentences/second)

- Parse error count (session and total)

- Last valid sentence timestamp

- Serial device path (from config, read-only display)

### 4.3.5 Display note — GPS coarse time offset

The GPS source in chrony sourcestats will show an offset of approximately -34ms. This is normal and expected: NMEA sentences are processed with an inherent delay and the GPS source is used only for coarse second-level time; PPS provides the precise sub-microsecond discipline. The dashboard must display a tooltip or footnote on the GPS source row explaining this, to avoid alarming users unfamiliar with the two-source (NMEA + PPS) timing model.

## Page 4 — Satellite View

Purpose: sky picture and per-satellite signal data. Most visually rich page.

### 4.4.1 Sky plot

- Polar chart: azimuth (0-360) x elevation (0-90)

- Each satellite plotted as a circle; position from azimuth/elevation NMEA data

- Circle colour: constellation (GPS = green, GLONASS = blue, BeiDou = amber). Galileo not active on this hardware.

- Circle size or fill: proportional to SNR / C/N0

- Satellites below elevation mask shown as hollow/greyed

- Satellites used in fix shown with filled circles; not-used as hollow

- Elevation mask ring drawn on chart (configurable angle)

### 4.4.2 Satellite table

- Columns: PRN, Constellation, Azimuth (deg), Elevation (deg), SNR (dBHz), Used in fix

- Sortable by any column

- Highlight row for satellite providing PPS reference if identifiable

### 4.4.3 Signal strength chart

- Horizontal bar chart of SNR for all visible satellites, sorted descending

- Colour coded by constellation

- Elevation mask threshold line shown

### 4.4.4 Rolling constellation chart

- Line chart: satellite count per constellation over last configurable window (default 1 hr)

- Reveals gradual obstructions or receiver reacquisition events

### 4.4.5 Layout note

Sky plot must be rendered full-width on mobile (square aspect ratio). Satellite table and bar chart stack below on mobile.

## Page 5 — System Health

Purpose: Pi hardware and OS metrics. Includes the temperature/frequency correlation which is diagnostically important for a timing server.

### 4.5.1 Stat cards

- CPU temperature (°C) — with configurable warning/critical thresholds

- CPU usage (%) — aggregate across cores

- Memory usage (used / total MB, percentage)

- Disk usage (root partition, percentage)

- Load average (1 min / 5 min / 15 min)

### 4.5.2 Service status indicators

- chronyd — running / stopped / error

- gpsd — running / stopped / error

- Web server process — running / stopped

- Poller process — running / last heartbeat timestamp

### 4.5.3 Charts

- CPU temperature over time

- CPU temperature vs frequency correction (ppm) — dual-axis overlay. Key diagnostic: thermal drift visible as correlation between temperature rises and frequency correction changes

- Memory and CPU usage over time

### 4.5.4 Network panel

- Primary interface: bytes in/out (rate, KB/s)

- NTP port (123) traffic indicator if measurable

- Hostname and IP address

## Page 6 — History & Trends

Purpose: long-term view using SQLite data. The page that justifies the persistence decision.

### 4.6.1 Time range selector

- Preset buttons: 24 hr / 7 d / 30 d / 90 d

- Custom date-range picker

### 4.6.2 Charts

- PPS offset over time (raw + smoothed overlay)

- Jitter over time

- Frequency correction (ppm) over time — long-term drift baseline

- Satellite count over time — reveals antenna obstructions or receiver issues

- CPU temperature over time

### 4.6.3 Daily summary table

- One row per day: date, mean offset, min offset, max offset, mean jitter, mean satellite count, any threshold breach count

- Exportable as CSV (download button)

## Page 7 — Configuration

Note: though we specified 5-6 pages, configuration has been separated into its own page to keep the layout clean. Total page count is seven including configuration.

Purpose: view and edit all configurable parameters. Changes write to config.json and are picked up by the poller on its next cycle (no restart required for most settings). A test/validate action is available before saving.

### 4.7.1 Data collection section

| Parameter | Details |
| --- | --- |
| Poll interval | Seconds. Presets: 5 / 10 / 30 / 60. Custom value allowed, minimum 5 s enforced. |
| Data retention | Days. Presets: 7 / 30 / 90 / 180 / 365. Older rows deleted automatically on nightly job. |
| Auto-purge | Toggle: enabled / disabled. If disabled, warns when database exceeds configurable size limit (MB). |
| Smoothing window | Minutes for rolling average used on stat card primary display. Options: 1 / 5 / 10. |
| Show raw + smoothed on charts | Toggle: show both traces / smoothed only. |
| Default chart time window | Time window loaded by default on detail pages. Options: 1 hr / 6 hr / 24 hr. |

### 4.7.2 Alert thresholds section

| Parameter | Details |
| --- | --- |
| Offset warning threshold | Nanoseconds. Default 100 ns. Overview card goes amber above this. |
| Offset critical threshold | Nanoseconds. Default 500 ns. Overview card goes red above this. |
| Jitter warning threshold | Nanoseconds. Default 50 ns. |
| Jitter critical threshold | Nanoseconds. Default 200 ns. |
| Root dispersion warning | Microseconds. Default 15 us. (Observed healthy value on PowerPi is ~11 us; 10 us default would cause spurious amber.) |
| Min satellites (warning) | Integer. Default 4. Fewer satellites in fix triggers amber. |
| Min satellites (critical) | Integer. Default 3. |
| CPU temp warning | Degrees C. Default 70. |
| CPU temp critical | Degrees C. Default 80. |
| Disk usage warning | Percentage. Default 80%. |
| Disk usage critical | Percentage. Default 90%. |
| Offset units on display | ns or us. Affects all pages. Default ns. |

### 4.7.3 GNSS / hardware section

| Parameter | Details |
| --- | --- |
| gpsd host | Hostname or IP. Default localhost. Test button connects and reads one TPV/SKY message. |
| gpsd port | Integer. Default 2947. |
| PPS device | Path to GPIO PPS device. Default /dev/pps0 (confirmed as GPIO pin 12 hardware pulse — chrony's active reference). pps1 = ptp0 (unrelated network clock), pps2 = ttyAMA0 (serial ldisc, lower quality). Used for status display and test action. |
| Elevation mask angle | Degrees. Default 10. Satellites below this angle are shown as inactive in sky plot and excluded from fix quality calculations. Should match receiver/gpsd elevation mask setting. |
| Expected constellations | Multi-select: GPS / GLONASS / BeiDou. Galileo not active on this hardware. Used to flag unexpected absence. Default: GPS + BeiDou. |

### 4.7.4 Chrony / NTP section

| Parameter | Details |
| --- | --- |
| chronyc binary path | Default /usr/bin/chronyc. Test button runs chronyc tracking and shows result. |
| Reference source label | Friendly name for the PPS reference, shown on all pages instead of raw reference ID. Default 'GPS PPS'. |
| Highlighted peers | Comma-separated list of peer IPs or hostnames to highlight in sourcestats table. |

### 4.7.5 UI / display section

| Parameter | Details |
| --- | --- |
| Site name | String. Shown in browser tab title and top navigation bar. Default 'Stratum-1 Dashboard'. |
| Default theme | Light / Dark / Follow system. Default: Follow system. |
| Default landing page | Which page loads at root URL. Default: Overview. |
| Show position on GNSS page | Toggle. If disabled, lat/lon/altitude are hidden (replaced by 'Hidden'). Default: enabled. |
| Auto-refresh interval (UI) | Seconds. How often pages poll the API. Default 10 s. Min 5 s. |

### 4.7.6 Test / validate actions

- Test chronyc — runs chronyc tracking, shows raw output in an expandable panel. Pass / fail badge.

- Test gpsd — connects to gpsd on configured host:port, reads one TPV (position) and one SKY (satellite) message, displays parsed summary. Pass / fail badge.

- Test PPS device — checks /dev/pps0 (and pps1, pps2) are present and accessible. Shows which pps devices exist. Pass / fail badge.

- Check database — shows SQLite file path, size, row counts per table, and oldest/newest sample timestamps.

### 4.7.7 Save behaviour

- Changes are written to config.json on Save

- Poller detects config file modification timestamp change and reloads on next poll cycle — no restart required

- A 'Reset to defaults' button restores all values to documented defaults after a confirmation prompt

# 5. Configuration Schema Reference

Full config.json key names and default values for developer reference.

| Key | Default | Notes |
| --- | --- | --- |
| poll_interval_seconds | 10 | Minimum 5 |
| retention_days | 90 |  |
| auto_purge | true |  |
| smoothing_window_minutes | 5 | Options: 1, 5, 10 |
| chart_show_raw | true | Show raw trace alongside smoothed |
| chart_default_window_hours | 1 |  |
| offset_warning_ns | 100 |  |
| offset_critical_ns | 500 |  |
| jitter_warning_ns | 50 |  |
| jitter_critical_ns | 200 |  |
| dispersion_warning_us | 15 | Revised from 10 — observed healthy value is ~11us |
| min_satellites_warning | 4 |  |
| min_satellites_critical | 3 |  |
| cpu_temp_warning_c | 70 |  |
| cpu_temp_critical_c | 80 |  |
| disk_warning_pct | 80 |  |
| disk_critical_pct | 90 |  |
| offset_display_unit | ns | ns or us |
| gpsd_host | localhost |  |
| gpsd_port | 2947 |  |
| pps_device | /dev/pps0 | GPIO 12 hardware PPS — confirmed as chrony reference source. pps1=ptp0 (unrelated), pps2=ttyAMA0 (serial ldisc, lower quality). |
| elevation_mask_degrees | 10 |  |
| expected_constellations | GPS,BeiDou | GLONASS monitored but optional. Galileo not active on this hardware. |
| chronyc_path | /usr/bin/chronyc |  |
| reference_label | GPS PPS | Friendly name for PPS source |
| highlighted_peers |  | Comma-separated IPs/hostnames |
| site_name | PowerPi Stratum-1 | Shown in browser tab and nav bar |
| hostname |  | Auto-populated from socket.gethostname() if blank. Override to set display name. |
| default_theme | system | light, dark, or system |
| default_landing_page | overview |  |
| show_position | true |  |
| ui_refresh_interval_seconds | 10 | Minimum 5 |

# 6. Assumptions and Constraints

- Network access: Dashboard is served on a trusted home LAN. No authentication, HTTPS, or rate limiting is required for v1.

- Single user: Config page has no locking. Concurrent edits from multiple browsers are not handled.

- Pi OS: Raspberry Pi OS (Debian-based). Python 3.11+, sqlite3 stdlib available. The sqlite3 CLI tool (separate package: apt install sqlite3) is required for database debugging and maintenance commands — not a runtime dependency.

- chrony: chronyc is available at the configured path and the running user has permission to call it without sudo.

- gpsd: gpsd is running on localhost:2947 and the Unix socket /run/gpsd.sock. The poller connects as a gpsd client using the gps Python library. gpsd started with -G (all interfaces), -n (no client needed to track), -b (read-only to receiver).

- PPS: GPIO PPS is confirmed active on GPIO pin 12 (not 18 as initially assumed). /dev/pps0 = GPIO 12 (hardware pulse, used by chrony — confirmed by sysfs name 'pps@12.-1' and live assert timestamps). /dev/pps1 = ptp0 (Pi network hardware clock, unrelated). /dev/pps2 = ttyAMA0 (serial line discipline PPS, lower quality). The pps_gpio kernel module is loaded and working. The dtoverlay entry in config.txt appears as bare filenames ('pps-gpio.dtbo', 'pps-rp1.dtbo') rather than proper dtoverlay= syntax — this is malformed but currently functional. Recommend correcting to 'dtoverlay=pps-gpio,gpiopin=12' at next maintenance opportunity.

- Constellations: LC76G actively reports GPS (gnssid 0) and BeiDou (gnssid 3). GLONASS sentences ($GLGSV) are emitted but satellites may not appear consistently in the gpsd SKY object. Galileo is not active. Dashboard displays what gpsd reports rather than assuming constellation availability.

- NMEA sentences confirmed: $GNGGA, $GNGSA, $GNRMC, $GNVTG, $GNZDA (multi-constellation combined), $GPGSV (GPS), $BDGSV (BeiDou), $GLGSV (GLONASS), $GPTXT (receiver text/antenna status). All data accessed via gpsd JSON protocol, not raw NMEA parsing.

- Chrony observed state: Reference ID 'PPS' (0x50505300), Stratum 1 confirmed. PPS source offset +3ns, std dev 309ns — headline metric for the dashboard. GPS source (coarse NMEA time) shows ~34ms offset which is normal and expected due to NMEA sentence processing delay; the dashboard must label this clearly to avoid alarming users. fritz.box NTP peer is present in chrony config but unreachable (0 samples) — sourcestats table must handle zero-sample peers gracefully. Chrony update interval is 8 seconds; poll interval default of 10s is appropriate.

- Root dispersion threshold: Observed root dispersion on a healthy server is ~11.257us. Default warning threshold revised to 15us (from 10us) to avoid spurious amber on a well-disciplined server.

- LC76G receiver: Command interface is disabled (confirmed in earlier hardware investigation). Dashboard is read-only with respect to the receiver — no configuration commands are sent.

- SQLite concurrency: Poller writes and web server reads operate on the same SQLite file. WAL mode will be enabled to allow concurrent read access without blocking writes.

- HTTP port: Web server runs on port 8080 by default. Not configurable via the UI to avoid self-lockout; change by editing the server startup command.

- Hostname: Default site hostname is PowerPi (from socket.gethostname()). Overridable via hostname key in config.json.

- Config file location: config.json lives alongside the application in a fixed known path. Not configurable.

- Notifications: Out of scope for v1. Config schema reserves a notifications block for v2 without requiring a schema migration.

# 7. Out of Scope (v1)

- Email or webhook alerting

- User authentication or access control

- HTTPS / TLS termination (use a reverse proxy if needed)

- Multi-server / multi-instance support

- Sending configuration commands to the GNSS receiver

- Leap second scheduling or manual chrony control from the UI

- Mobile app or PWA packaging

# 8. Open Questions — All Resolved

All open questions raised during requirements definition have been resolved. Resolutions documented below for traceability.

| # | Resolution |
| --- | --- |
| OQ-1 — PPS GPIO pin | RESOLVED — CORRECTED. GPIO PPS is active on GPIO pin 12 (not 18 as initially assumed, and not serial line discipline only as previously concluded). Confirmed via sysfs: /dev/pps0 name = 'pps@12.-1' with live assert timestamps. pps_gpio kernel module loaded. dtoverlay entry in config.txt is malformed (bare filenames rather than dtoverlay= syntax) but functional. Config default: pps_device = /dev/pps0. Recommend correcting config.txt to 'dtoverlay=pps-gpio,gpiopin=12' at next maintenance. |
| OQ-2 — gpsd vs direct serial | RESOLVED: gpsd is the primary interface. Running as: gpsd -G -n -b /dev/serial0 /dev/pps0. Poller will use the gps Python client library connecting to localhost:2947. Direct serial parsing is not required. |
| OQ-3 — NMEA sentence set | RESOLVED: Confirmed sentences: $GNGGA, $GNGSA, $GNRMC, $GNVTG, $GNZDA (multi-constellation), $GPGSV (GPS), $BDGSV (BeiDou), $GLGSV (GLONASS), $GPTXT (antenna/firmware text). All data accessed via gpsd JSON protocol (TPV, SKY, ATT classes), not raw NMEA parsing. |
| OQ-4 — Active constellations | RESOLVED: GPS (gnssid 0) and BeiDou (gnssid 3) confirmed active in gpsd SKY object. 15 satellites in view, 9 used in fix observed. GLONASS sentences emitted by receiver but satellites not consistently appearing in SKY object — monitored but not in default expected set. Galileo not active. No QZSS observed. |
| OQ-5 — Web server | RESOLVED: Flask for v1. Single process, simpler deployment, supports background poller thread. Revisit if robustness requirements increase. |
| OQ-6 — Sky plot library | RESOLVED: Custom SVG rendered in the browser from azimuth/elevation JSON data. No additional charting library dependency. Allows full control over constellation colouring, elevation mask ring, and SNR encoding. Can be migrated to D3 in a future iteration without changing the data API. |

# 9. Project Structure and Setup

## 9.1 Directory layout

All project files live under /opt/stratum1-dashboard/ (INSTALL_DIR). The project uses a clean four-directory separation by concern.

| Path | Type | Purpose |
| --- | --- | --- |
| app/paths.py | Python | Central path definitions. All modules import paths from here. Respects STRATUM1_DIR env var. |
| app/app.py | Python | Flask application entry point. Starts poller thread, registers all API blueprints. |
| app/poller.py | Python | Background daemon thread. Polls all collectors on interval. Writes to SQLite. Maintains ring buffer. |
| app/config.py | Python | Load/save/validate config.json from data/. Provides defaults. Watches mtime for live reload. |
| app/database.py | Python | SQLite schema (WAL mode), write helpers, aggregation queries, retention purge, heartbeat. |
| app/collectors/chrony.py | Python | Parses chronyc tracking, sourcestats, sources -v into structured dicts. |
| app/collectors/gpsd_client.py | Python | gpsd JSON protocol client. Reads TPV (position) and SKY (satellites) messages. |
| app/collectors/system.py | Python | CPU temp/usage, memory, disk, load average, service status, PPS device check. |
| app/api/overview.py | Python | Flask blueprint — GET /api/overview |
| app/api/chrony.py | Python | Flask blueprint — GET /api/chrony?range=Nh |
| app/api/gnss.py | Python | Flask blueprint — GET /api/gnss |
| app/api/satellites.py | Python | Flask blueprint — GET /api/satellites |
| app/api/system.py | Python | Flask blueprint — GET /api/system |
| app/api/history.py | Python | Flask blueprint — GET /api/history?range=Nh¦Nd |
| app/api/config_api.py | Python | Flask blueprints — GET/POST /api/config, POST /api/config/test |
| web/templates/base.html | HTML | Tabler base layout: sidebar nav, topbar, dark/light theme toggle, shared JS helpers. |
| web/templates/overview.html | HTML | Page 1 — stat cards, sparklines, status detail, alerts panel. |
| web/templates/chrony.html | HTML | Page 2 — tracking panel, sources table, offset/frequency charts. |
| web/templates/gnss.html | HTML | Page 3 — fix status, DOP, constellation bars, position. |
| web/templates/satellites.html | HTML | Page 4 — custom SVG sky plot, satellite table, SNR bars, count chart. |
| web/templates/system.html | HTML | Page 5 — stat cards, services, PPS devices, temp/freq dual chart. |
| web/templates/history.html | HTML | Page 6 — long-range charts, daily summary table, CSV export. |
| web/templates/config.html | HTML | Page 7 — settings form, test action buttons, save/reset. |
| web/static/css/tabler.min.css | CSS | Tabler v1.0.0-beta17 stylesheet (local copy). |
| web/static/js/tabler.min.js | JS | Tabler JS bundle (local copy). |
| web/static/js/apexcharts.min.js | JS | ApexCharts v3.49.0 (local copy — no CDN dependency at runtime). |
| data/config.json | JSON | Runtime configuration. Auto-created with defaults on first run. Edit here or via /config page. |
| data/stratum1.db | SQLite | Metrics database. Auto-created on first run. WAL mode. Back up this directory. |
| docs/ | Dir | Project documentation: requirements spec, setup guide, operations guide. |
| venv/ | Dir | Python virtual environment. Never edit directly. |
| requirements.txt | Text | Python dependencies for pip install. |
| stratum1-dashboard.service | systemd | systemd unit file. Copied to /etc/systemd/system/ during setup. |

## 9.2 Python virtual environment

A dedicated venv isolates the project from system Python and makes dependency management explicit.

Setup commands (run once on the Pi):

sudo mkdir -p /opt/stratum1-dashboard

sudo chown pi:pi /opt/stratum1-dashboard

cd /opt/stratum1-dashboard

python3 -m venv venv

source venv/bin/activate

pip install -r requirements.txt

## 9.3 Python dependencies (requirements.txt)

| Package | Version | Purpose |
| --- | --- | --- |
| flask | >=3.0 (confirmed: 3.1.3) | Web framework. Serves HTML pages and JSON API endpoints. |
| gpsd-py3 | >=0.3 (confirmed: 0.3.0) | Pure Python gpsd client. Connects to localhost:2947, reads TPV and SKY messages. |
| psutil | >=5.9 (confirmed: 7.2.2) | System stats: CPU temp, usage, memory, disk, load average, process status. |
| schedule | >=1.2 (confirmed: 1.2.2) | Lightweight job scheduler used by the poller for timed collection cycles. |

Note: sqlite3 and subprocess are Python stdlib — no pip install required. The sqlite3 CLI tool for inspecting the database directly is a separate system package (sudo apt install sqlite3) and is not installed by default on Pi OS; add it to the system prerequisites step. The gps package from gpsd itself is an alternative to gpsd-py3; gpsd-py3 is preferred as it is a pure Python client with no C dependency.

## 9.4 systemd service

For automatic startup and restart on failure, the dashboard runs as a systemd user service. The unit file (stratum1-dashboard.service) is included in the project directory and symlinked into /etc/systemd/system/. Key settings: restart on failure with 5s delay, runs as user pi, working directory set to project root, venv Python used as ExecStart interpreter.

## 9.5 Build order

Recommended implementation sequence to allow incremental testing at each stage:

- Stage 0 — paths.py. Central path module. Tested with python3 app/paths.py — confirms ROOT, DATA_DIR, TEMPLATES_DIR all resolve correctly.

- Stage 1 — config.py + config.json defaults. No dependencies. Tested with python3 app/config.py — creates data/config.json with all defaults.

- Stage 2 — database.py. Schema creation, WAL mode, write/read helpers. Tested with python3 app/database.py — creates data/stratum1.db. Verify with: sqlite3 data/stratum1.db '.tables'

- Stage 3 — collectors/chrony.py. Parses chronyc tracking, sourcestats, sources -v. Tested with python3 app/collectors/chrony.py — uses embedded sample data if chronyc unavailable.

- Stage 4 — collectors/gpsd_client.py. Connects to gpsd:2947, reads TPV and SKY. Tested with python3 app/collectors/gpsd_client.py — requires live gpsd.

- Stage 5 — collectors/system.py. CPU, memory, disk, services, PPS devices. Tested with python3 app/collectors/system.py — Pi-specific assertions gated on hardware detection.

- Stage 6 — poller.py. Background thread tying all collectors. Tested with python3 app/poller.py --once — one cycle, verify DB rows with sqlite3.

- Stage 7 — app.py + 7 API blueprints. Flask app starts poller thread, registers blueprints. Tested with python3 app/app.py then curl http://localhost:8080/api/overview.

- Stage 8 — web/templates/base.html + overview.html. First visual milestone. Confirms full pipeline. Open http://PowerPi:8080/ in browser.

- Stage 9 — Remaining 6 pages: chrony.html, gnss.html, satellites.html (custom SVG sky plot), system.html, history.html, config.html. Built and tested one at a time.

# 10. Database Metric Reference

Canonical metric names written to raw_samples by poller.py. These are the keys used in all API queries and database operations.

## 10.1 Source: chrony

| Metric | Unit | Description |
| --- | --- | --- |
| pps_offset_ns | ns | PPS adjusted offset from chronyc sources -v (primary timing metric) |
| rms_offset_ns | ns | RMS offset from chronyc tracking |
| last_offset_ns | ns | Last measured offset from chronyc tracking |
| freq_ppm | ppm | Frequency correction (positive = slow) |
| residual_freq_ppm | ppm | Residual frequency from chronyc tracking |
| skew_ppm | ppm | Frequency skew |
| root_delay_ns | ns | Root delay |
| root_dispersion_ns | ns | Root dispersion in nanoseconds |
| root_dispersion_us | us | Root dispersion in microseconds (used for threshold evaluation) |
| update_interval_s | s | Chrony update interval |
| stratum |  | NTP stratum (float, always 1.0 for this server) |
| reference_name | string | Reference source name (e.g. PPS) |
| leap_status | string | Leap second status (e.g. Normal) |

## 10.2 Source: gnss

| Metric | Unit | Description |
| --- | --- | --- |
| fix_mode |  | gpsd fix mode integer (0=Unknown, 1=NoFix, 2=2D, 3=3D) |
| fix_mode_name | string | Fix mode as string (e.g. 3D) |
| latitude | deg | Decimal degrees — only stored if show_position=true in config |
| longitude | deg | Decimal degrees — only stored if show_position=true in config |
| altitude_m | m | Altitude above WGS84 ellipsoid in metres |
| sats_visible | count | Total satellites in view |
| sats_used | count | Satellites used in current fix |
| hdop |  | Horizontal dilution of precision |
| vdop |  | Vertical dilution of precision |
| pdop |  | Position dilution of precision |

## 10.3 Source: system

| Metric | Unit | Description |
| --- | --- | --- |
| cpu_temp_c | degC | CPU temperature from /sys/class/thermal/thermal_zone0/temp |
| cpu_usage_pct | pct | Aggregate CPU usage percentage |
| cpu_freq_mhz | MHz | Current CPU frequency |
| mem_usage_pct | pct | Memory usage percentage |
| disk_usage_pct | pct | Root filesystem usage percentage |
| load_1 |  | 1-minute load average |
| load_5 |  | 5-minute load average |
| load_15 |  | 15-minute load average |
| uptime_seconds | s | System uptime in seconds |
| chronyd_status | string | systemd service status: active ¦ inactive ¦ failed ¦ unknown |
| gpsd_status | string | systemd service status: active ¦ inactive ¦ failed ¦ unknown |

# 11. Revision History

| Version | Notes |
| --- | --- |
| v0.1 — Draft | Initial requirements captured from design discussion. All sections subject to review before coding begins. |
| v0.2 — Draft | All open questions resolved. PPS via serial line discipline confirmed (no GPIO). gpsd confirmed as NMEA interface. Constellation set updated: GPS + BeiDou active, GLONASS monitored, Galileo not active. Config schema updated: gpio pin removed, gpsd host/port and pps_device added, hostname added, expected_constellations default updated. Assumptions section updated throughout. Sky plot approach confirmed as custom browser SVG. |
| v0.5 — Final | Project implementation complete. Section 9 updated to final directory structure (app/, web/, data/, docs/). Build order updated to include Stage 0 (paths.py). Confirmed package versions added. Section 10 (Database Metric Reference) added with canonical metric names for all three sources. Setup commands updated to /opt/stratum1-dashboard. All pages built and verified on PowerPi hardware. systemd service confirmed working with auto-restart on reboot. |
| v0.4 — Draft | sqlite3 CLI added to system prerequisites. Confirmed not installed by default on Pi OS Bookworm — must be installed separately (sudo apt install sqlite3). Required for Stage 2 database verification and Section 7 maintenance commands. Not a runtime dependency. |
| v0.3 — Draft | GPIO PPS on pin 12 confirmed (corrects v0.2 conclusion of serial-ldisc-only). Three PPS devices identified: pps0=GPIO12 (active chrony reference), pps1=ptp0 (network clock, unrelated), pps2=ttyAMA0 (serial ldisc). config.txt dtoverlay entries malformed but functional — maintenance recommendation added. Root dispersion warning threshold revised 10us->15us based on observed healthy server value (~11us). Chrony live data captured: PPS +3ns/309ns std dev, GPS -34ms (expected, NMEA delay). GPS offset display note and fritz.box zero-sample handling added to page specs. Galileo removed from constellation colour key. Project structure, venv setup, dependencies, systemd service, and build order added as Section 9. |