Stratum-1 Dashboard

Operations Guide

Day-to-day reference for maintaining and monitoring the dashboard

| Hardware | Raspberry Pi 5 — PowerPi |
| --- | --- |
| Install path | /opt/stratum1-dashboard |
| Service | stratum1-dashboard.service |
| Dashboard URL | http://PowerPi:8080/ |
| Health endpoint | http://PowerPi:8080/health |
| Config file | /opt/stratum1-dashboard/data/config.json |
| Database | /opt/stratum1-dashboard/data/stratum1.db |
| Logs | journalctl -u stratum1-dashboard |

# 1. Quick Reference

| Task | Command |
| --- | --- |
| Check service status | sudo systemctl status stratum1-dashboard |
| Start the service | sudo systemctl start stratum1-dashboard |
| Stop the service | sudo systemctl stop stratum1-dashboard |
| Restart the service | sudo systemctl restart stratum1-dashboard |
| View live logs | journalctl -u stratum1-dashboard -f |
| View last 50 log lines | journalctl -u stratum1-dashboard -n 50 |
| Check PPS offset now | chronyc tracking ¦ grep 'System time' |
| Check chrony sources | chronyc sources -v |
| Check all services | sudo systemctl status chronyd gpsd stratum1-dashboard |
| Database size | du -sh /opt/stratum1-dashboard/data/ |
| Database row count | sqlite3 /opt/stratum1-dashboard/data/stratum1.db 'SELECT COUNT(*) FROM raw_samples;' |
| Backup data | tar -czf stratum1-data-$(date +%Y%m%d).tar.gz -C /opt/stratum1-dashboard data/ |
| Open config page | http://PowerPi:8080/config |

# 2. Reading the Dashboard

## 2.1 Overview page — status badge

| Badge | Meaning |
| --- | --- |
| ● OK (green) | All metrics within thresholds. Server is well-disciplined. |
| ▲ WARN (amber) | One or more metrics above warning threshold but below critical. Server is functional but worth investigating. |
| ✕ ALERT (red) | One or more metrics above critical threshold. Action may be required. |
| — (grey) | No data yet — poller has not completed its first cycle. |

Note: On first start or after a reboot, the badge will show ALERT for 1-3 minutes while chrony re-acquires PPS lock. This is normal. The badge will transition to WARN then OK automatically.

## 2.2 NTP/Chrony page — sources table

| Symbol | Meaning |
| --- | --- |
| # * (green highlight) | PPS source — current best reference. This should always have * state on a healthy server. |
| # ? GPS row | GPS (NMEA) source — always shows ? state and a large negative offset (~10-40ms). This is normal. The NMEA delay is expected; PPS provides precise timing. |
| ^ - | NTP server peer — combined in clock selection but not current best. |
| ^ ? fritz.box | fritz.box NTP — always shows ? with dashes. The home router NTP is unreachable or too noisy. Normal. |
| * state | Current best source (should be PPS). |
| + state | Combined — contributing to clock but not primary. |
| - state | Not combined — available but not used. |
| ? state | Unusable — bad reach register, too variable, or never received. |

## 2.3 Satellites page — sky plot

- Filled circles with PRN number: satellite used in current fix

- Hollow coloured circles: satellite visible but not used in fix

- Grey/hollow near edge: satellite below elevation mask (10°)

- Dashed red ring: elevation mask boundary

- Circle size: proportional to SNR (larger = stronger signal)

- Green = GPS, Orange = BeiDou, Blue = GLONASS, Purple = Galileo

Note: GLONASS satellites appear intermittently depending on geometry. BeiDou satellites are often visible but show 0 used — this is normal when GPS provides sufficient fix geometry.

## 2.4 System Health page — temp/frequency chart

The dual-axis chart on the System Health page shows CPU temperature (orange, left axis) and chrony frequency correction (blue dashed, right axis) on the same time axis. These two traces should show correlation — when temperature rises, frequency correction changes in tandem. This is thermal drift: the crystal oscillator's frequency changes slightly with temperature. A well-insulated Pi in a stable environment will show small, slow variations. Rapid temperature spikes (e.g. from a burst of computation) will show corresponding frequency spikes.

# 3. Normal Operating Values

The following ranges are observed on PowerPi under normal conditions. Values outside these ranges are not necessarily errors, but are worth investigating.

| Metric | Normal range |
| --- | --- |
| PPS offset (disciplined) | ±10 ns to ±300 ns |
| PPS offset (settling after reboot) | Up to ±1,000,000 ns — reduces over 1-3 min |
| Jitter (RMS offset) | 100 ns to 1,500 ns |
| Root dispersion | 8 µs to 15 µs |
| Frequency correction | 4.5 to 5.5 ppm slow (varies slowly with temperature) |
| Skew | 0.05 to 0.20 ppm |
| GPS source offset | -5 ms to -40 ms (NMEA delay — always negative, always large) |
| CPU temperature (idle) | 44°C to 56°C |
| CPU usage | 0% to 5% (dashboard adds negligible load) |
| Memory usage | 8% to 15% |
| Satellites visible | 6 to 20 (varies through day with geometry) |
| Satellites used | 4 to 12 |
| HDOP | 0.8 to 2.0 (lower is better) |

# 4. Common Situations

## 4.1 Dashboard shows ALERT after power cut / reboot

Expected. Chrony needs 1-3 minutes to re-acquire PPS lock after a restart. Wait and watch the PPS offset on the Overview page decrease. The badge will go WARN then OK automatically. No action required unless it does not settle within 5 minutes.

If it does not settle, check:

chronyc sources -v   # Is PPS source showing * state?

chronyc tracking     # Is system time offset decreasing?

sudo systemctl status gpsd   # Is gpsd running?

ls -la /dev/pps*     # Are PPS devices present?

## 4.2 PPS offset stuck at a large value

If PPS offset is large and not reducing after 5 minutes, chrony may have lost its PPS reference.

chronyc sources -v

# Check if PPS row has * state

# If PPS shows ? state, PPS signal may be lost

# Check PPS device is present and firing

cat /sys/class/pps/pps0/assert

# Should show a recent timestamp updating every second

If /sys/class/pps/pps0/assert is not updating, the GNSS module may have lost fix. Check the GNSS Status page — if fix mode shows No fix, the antenna may have been obstructed.

## 4.3 Status badge stays WARN — offset above threshold

The 5-minute rolling mean is above the warning threshold (100ns). This may be normal during settling, or may indicate genuine degradation. Check:

- NTP/Chrony page: Is PPS still the current best source (* state)?

- Satellites page: Is satellite count lower than usual? Poor geometry increases offset variance.

- System Health page: Is CPU temperature unusually high? Thermal instability increases frequency drift.

- Time of day: Satellite geometry varies — some periods naturally produce wider offset variation.

Note: If the server has been running for more than 10 minutes and PPS is locked with * state, a WARN badge with offset 100-300ns is not a problem. The warning threshold is intentionally low to flag genuine issues.

## 4.4 Dashboard not loading after reboot

sudo systemctl status stratum1-dashboard

journalctl -u stratum1-dashboard -n 30

# Common causes:

# - gpsd not started yet (service dependency timing)

# - Port 8080 already in use

# - Python path issue

If the service failed, check the log output and restart:

sudo systemctl restart stratum1-dashboard

sudo systemctl status stratum1-dashboard

## 4.5 No satellite data / sky plot empty

# Check gpsd is running

sudo systemctl status gpsd

# Test gpsd directly

gpspipe -w -n 5

# Should print JSON TPV and SKY messages

# Check antenna connection

# SKY message with 0 satellites = antenna fault or obstruction

## 4.6 History page shows no data / sparse data

The History page requires data to have accumulated in the database. On first deployment, only the current day's data is available. After 24 hours, the 24h view fills in. After 7 days, the 7d view becomes meaningful.

If data appears to stop at a point in time, the poller may have crashed. Check:

journalctl -u stratum1-dashboard --since '1 hour ago'

# Look for ERROR or WARNING lines

# Check database heartbeat age

# Config page → Check database → see Heartbeat age

# 5. Updating the Application

When new Python or template files are available (e.g. bug fixes or new features):

- Stop the service: sudo systemctl stop stratum1-dashboard

- Copy changed files to their correct subdirectory (Python → app/, templates → web/templates/, static assets → web/static/)

- If requirements.txt changed: source venv/bin/activate && pip install -r requirements.txt

- Restart: sudo systemctl start stratum1-dashboard

- Verify: sudo systemctl status stratum1-dashboard && curl -s http://localhost:8080/health

Important: Never delete data/stratum1.db or data/config.json during an update. Only files in app/ and web/ change between versions. The data/ directory is never touched by an update.

# 6. Configuration

All settings are managed via the Configuration page at http://PowerPi:8080/config. Changes take effect on the next poller cycle (within poll_interval_seconds) without a service restart.

## 6.1 Adjusting alert thresholds

If the dashboard shows persistent WARN/ALERT due to thresholds that don't match your server's actual performance, adjust them on the Configuration page under Alert Thresholds. For example, if your server routinely achieves ±50ns, tightening the offset warning to 75ns gives more meaningful alerts.

## 6.2 Hiding position

If you do not want latitude/longitude displayed (e.g. for privacy), set show_position to Hidden on the Configuration page. Altitude is still shown. The poller stops storing lat/lon in the database when this is set.

## 6.3 Adjusting poll interval

The default 10s poll interval is appropriate for most uses. Increasing it (e.g. to 30s) reduces database growth at the cost of less granular sparklines. Note that chrony's own update interval is 8s, so polling faster than that gives diminishing returns.

## 6.4 Config page test actions

| Test action | What it verifies |
| --- | --- |
| Test chronyc | Runs chronyc tracking live. Confirms binary path and permissions. Shows raw output. |
| Test gpsd | Connects to gpsd, reads one TPV+SKY. Shows fix mode and constellation summary. Confirms gpsd is reachable. |
| Test PPS devices | Lists /dev/pps0-2 with sysfs names. Confirms GPIO PPS device (pps@12.-1) is present. |
| Check database | Shows file path, size, row counts, oldest/newest samples, and poller heartbeat age. |

# 7. Database Maintenance

## 7.1 Checking database size

du -sh /opt/stratum1-dashboard/data/

du -sh /opt/stratum1-dashboard/data/stratum1.db

# Row count

sqlite3 /opt/stratum1-dashboard/data/stratum1.db \

'SELECT COUNT(*) FROM raw_samples;'

# Data date range

sqlite3 /opt/stratum1-dashboard/data/stratum1.db \

'SELECT MIN(datetime(timestamp,"unixepoch")),

MAX(datetime(timestamp,"unixepoch")) FROM raw_samples;'

## 7.2 Expected database growth

At 34 metrics per cycle and a 10s poll interval, the database accumulates approximately 12,000 rows per hour, 290,000 rows per day, and 26 million rows over 90 days. In practice, the database stays well under 500MB for the default 90-day retention period.

## 7.3 Manual purge

The auto-purge runs automatically at midnight. To run it manually:

# From the venv Python

cd /opt/stratum1-dashboard

source venv/bin/activate

python3 -c "

import sys; sys.path.insert(0, 'app')

import database, config

config.load()

database.init()

n = database.purge_old_data(config.get('retention_days', 90))

print(f'Deleted {n} rows')

"

## 7.4 Backup

# Data only (config + database) — most important

tar -czf ~/stratum1-data-$(date +%Y%m%d).tar.gz \

-C /opt/stratum1-dashboard data/

# Full project backup (excluding venv)

tar -czf ~/stratum1-full-$(date +%Y%m%d).tar.gz \

-C /opt stratum1-dashboard/ --exclude=venv

## 7.5 Restoring from backup

sudo systemctl stop stratum1-dashboard

# Restore data directory

tar -xzf stratum1-data-20260512.tar.gz \

-C /opt/stratum1-dashboard/

sudo systemctl start stratum1-dashboard

# 8. Log Reference

## 8.1 Viewing logs

# Live log stream (Ctrl+C to exit)

journalctl -u stratum1-dashboard -f

# Last 50 lines

journalctl -u stratum1-dashboard -n 50

# Since last boot

journalctl -u stratum1-dashboard -b

# Between times

journalctl -u stratum1-dashboard \

--since '2026-05-12 10:00' --until '2026-05-12 11:00'

## 8.2 Log message reference

| Log message | Meaning / action |
| --- | --- |
| INFO Database initialised. WAL mode enabled. | Normal startup. Database ready. |
| INFO Poller thread started (interval=10s) | Normal startup. Poller running. |
| INFO Dashboard listening on http://0.0.0.0:8080 | Flask server started. Dashboard accessible. |
| DEBUG Cycle N complete — 34 samples written | Normal poll cycle. All collectors succeeded. |
| WARNING chronyc tracking failed: ... | chronyc command failed. Check chrony is running and chronyc_path is correct. |
| WARNING gpsd collect failed: Connection refused | gpsd is not running or not on configured host:port. |
| WARNING Cycle completed with errors: ... | One or more collectors failed on this cycle. Data may be partial. |
| INFO Config file changed — reloading | config.json was modified. Poller picked up the new settings. |
| INFO Nightly purge: deleted N rows older than 90 days | Normal nightly maintenance. |
| ERROR Database write failed: ... | SQLite write error. Check disk space and database file permissions. |

# 9. Hardware Notes

## 9.1 PPS devices

| Device | Identity / notes |
| --- | --- |
| /dev/pps0 | GPIO pin 12 hardware PPS. This is the chrony reference source. sysfs name: pps@12.-1. The primary timing device. |
| /dev/pps1 | ptp0 — Raspberry Pi 5 hardware network clock. Not related to GNSS. |
| /dev/pps2 | ttyAMA0 — Serial line discipline PPS from GNSS UART. Lower quality than GPIO PPS. Not used by chrony. |

Note: The dtoverlay entry in /boot/firmware/config.txt for pps-gpio is present as a bare filename (pps-gpio.dtbo) rather than the correct dtoverlay= syntax. This is a known quirk — it is functional but should be corrected to 'dtoverlay=pps-gpio,gpiopin=12' at the next maintenance opportunity.

## 9.2 GNSS module — LC76G (Waveshare Pico-GPS-L76X HAT)

- Chip: Quectel LC76G (substituted for the labelled L76X)

- Command interface: disabled — module is read-only from the Pi's perspective

- Active constellations: GPS, BeiDou, GLONASS (intermittent). Galileo not active.

- NMEA sentences: $GNGGA, $GNGSA, $GNRMC, $GNVTG, $GNZDA, $GPGSV, $BDGSV, $GLGSV, $GPTXT

- gpsd interface: /dev/serial0, port 2947, started with -G (all interfaces) -n (always track) -b (read-only)

## 9.3 Chrony configuration

- Reference: PPS (Reference ID 50505300, decoded as 'PPS')

- Stratum: 1

- Update interval: 8 seconds

- Typical PPS offset when disciplined: ±10–300 ns

- Typical root dispersion: 8–15 µs

- NTP peers: 4 stratum-2 servers + fritz.box (unusable)

- GPS source: provides coarse NMEA time only (~-30ms offset). PPS provides precise discipline.

# 10. Revision History

| Version | Notes |
| --- | --- |
| v1.0 | Initial release. Covers PowerPi / Raspberry Pi 5 with LC76G GNSS, GPIO 12 PPS, gpsd on localhost:2947, chrony Stratum 1. All seven dashboard pages verified. systemd service confirmed with auto-restart on reboot. |